NON-WSQ COURSE

LEARNER GUIDE

For

Arduino Masterclass

Course Code: C218

Conducted by Tertiary Infotech Academy Pte Ltd

UEN: 201200696W

Version 1.0

# Document Version Control Record

| Version Number | Effective Date of Release | Summary of Included Changes | Author |
| --- | --- | --- | --- |
| 1.0 | 30 September 2026 | First version | Tertiary Infotech Academy Pte Ltd |

# Contents

Course Overview

Learning Outcomes

Safety and Toolchain

Topic 1: Introduction to Embedded Systems

Lab 1: Board Check and Deterministic Blink

Lab 2: Serial Command Control

Topic 2: Introduction to Basic Electronics

Lab 3: Safe LED Circuit and Ohm's Law

Lab 4: Push Button with INPUT_PULLUP

Lab 5: PWM Fade with Non-blocking Timing

Topic 3: Analogue Sensors and Transducers

Lab 6: LDR Dark-Activated Lamp with Hysteresis

Lab 7: Potentiometer-Controlled Servo

Topic 4: Digital Sensors and Transducers

Lab 8: Ultrasonic Distance with Timeout

Lab 9: PIR Motion-Activated Light

Topic 5: Actuators

Lab 10: DHT11 Temperature and Humidity Monitor

Lab 11: Stepper Motor Position Control

Lab 12: DHT Fan Controller with Hysteresis

Technical References

# Course Overview

This guide develops one continuous embedded-control stack: sense a physical condition, condition and acquire the signal, make a bounded decision, drive a protected actuator, and verify the result with observable evidence. It retains the approved five-topic coverage from the original v12 course while replacing legacy overview-only treatment with current Arduino mechanisms, calculations, code contracts, failure analysis, and acceptance tests.

| Course code | C218 |
| --- | --- |
| Duration | One day: 7.5 instructional hours (9:30am - 5:30pm) |
| Course page | https://www.tertiarycourses.com.sg/arduino-masterclass.html |
| Arduino reference | https://docs.arduino.cc/language-reference/ |

# Learning Outcomes

LO1 - Analyse real-life applications using Arduino microcontroller

LO2 - Implement basic electrical circuits using Arduino microcontroller

LO3 - Apply Arduino microcontroller to analogue sensors and transducers

LO4 - Apply Arduino microcontroller to digital sensors and transducers

LO5 - Implement actuators with Arduino microcontroller

# Safety and Toolchain

Disconnect power before rewiring or continuity testing.

Never power a motor or servo from an Arduino I/O pin. Use a suitable driver and supply.

Use a flyback diode for inductive loads and establish a common ground.

Verify board and serial port before upload; use a known USB data cable.

Keep conductors clear and stop immediately if a component overheats, smells, or the board repeatedly resets.

# Topic 1: Introduction to Embedded Systems

An embedded system is a bounded control system. Its architecture, memory, timing, I/O and evidence must support a dedicated physical function. On reset, the bootloader and sketch initialise hardware before loop() executes repeatedly. Serial communication provides both a command path and a diagnostic trace.

## Slide alignment

Slides 10-27 | Skill focus: Execute performance specification and analysis of sensors and actuators for real-life applications | Key concepts: Underlying concepts pertaining to performance specification and analysis

## Mechanism checklist

Controller resources and boundaries

setup()/loop() runtime

blocking vs non-blocking timing

UART command framing and evidence

## Lab 1: Board Check and Deterministic Blink

| Alignment | LO1 |
| --- | --- |
| Time | 15 minutes |
| Objective | Validate the Arduino Uno/Maker UNO toolchain and prove deterministic digital output timing. |
| Deliverable | A board-identification record and a 250 ms LED heartbeat that verifies compile, upload, reset, and loop execution. |
| Folder | labs/lab-01-board-check-blink/ |

### Components

Arduino Uno or Maker UNO

USB data cable

Arduino IDE 2.x

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| LED_BUILTIN | On-board LED | No external wiring |
| USB | Computer | Power + serial upload |

### Detailed procedure

Connect the board using a USB data cable and identify the board/port shown by the IDE.

Open the starter sketch and predict the LED period before compiling.

Select Arduino Uno and the detected serial port, then compile and upload.

Time ten complete blink cycles and compare the observed period with the expected 500 ms cycle.

Change INTERVAL_MS to 1000, upload again, and record the changed evidence.

### Solution sketch

const uint8_t LED_PIN = LED_BUILTIN; const unsigned long INTERVAL_MS = 250; void setup() { pinMode(LED_PIN, OUTPUT); } void loop() { digitalWrite(LED_PIN, HIGH); delay(INTERVAL_MS); digitalWrite(LED_PIN, LOW); delay(INTERVAL_MS); }

### Acceptance checks

Sketch compiles with no error

Upload completes on the selected port

LED cycles 250 ms ON and 250 ms OFF

Measured ten-cycle duration is approximately 5 seconds

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Port missing | Try a known USB data cable, reconnect, and confirm the CH340/USB serial device. |
| Upload timeout | Check Board and Port, close Serial Monitor, then press reset before retrying. |
| Wrong timing | Confirm milliseconds and that both HIGH and LOW paths use INTERVAL_MS. |

## Lab 2: Serial Command Control

| Alignment | LO1 |
| --- | --- |
| Time | 15 minutes |
| Objective | Trace UART data from a typed ASCII command to a validated output state. |
| Deliverable | A line-oriented command parser that accepts ON, OFF, and STATUS without blocking indefinitely. |
| Folder | labs/lab-02-serial-command-control/ |

### Components

Arduino Uno or Maker UNO

USB data cable

Arduino IDE Serial Monitor

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| LED_BUILTIN | On-board LED | Output |
| USB serial | Computer | 9600 bit/s, newline terminator |

### Detailed procedure

Upload the starter sketch and open Serial Monitor at 9600 baud with Newline selected.

Send ON, OFF, and STATUS while observing both the LED and acknowledgement text.

Send a lowercase command and an invalid command; record how the parser handles each.

Disconnect and reconnect the board, then confirm the default output is safe and OFF.

### Solution sketch

String command; bool ledOn = false; void applyOutput() { digitalWrite(LED_BUILTIN, ledOn ? HIGH : LOW); } void setup() { pinMode(LED_BUILTIN, OUTPUT); applyOutput(); Serial.begin(9600); Serial.println("READY: ON ¦ OFF ¦ STATUS"); } void loop() { if (Serial.available() == 0) return; command = Serial.readStringUntil('\n'); command.trim(); command.toUpperCase(); if (command == "ON") ledOn = true; else if (command == "OFF") ledOn = false; else if (command == "STATUS") { /* report only */ } else { Serial.println("ERR"); return; } applyOutput(); Serial.println(ledOn ? "LED=ON" : "LED=OFF"); }

### Acceptance checks

ON lights the LED

OFF extinguishes the LED

STATUS reports the true output state

Unknown input returns ERR without changing output

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Garbled text | Set Serial Monitor to 9600 baud to match Serial.begin(). |
| Commands ignored | Select Newline and confirm the parser removes carriage return/newline characters. |
| LED state wrong | Read back the maintained ledOn state rather than assuming the command succeeded. |

# Topic 2: Introduction to Basic Electronics

Basic electronics turns code into safe physical behaviour. Ohm's law sets current limits, breadboard topology determines real connectivity, pull resistors prevent floating inputs, debounce rejects switch transitions, and PWM changes duty cycle while a driver protects the microcontroller from higher-current loads.

## Slide alignment

Slides 28-46 | Skill focus: Utilise commonly adopted component interconnections and signal-conditioning principles in automation | Key concepts: Implementation of component interconnections and signal conditioning concepts

## Mechanism checklist

Ohm's law and safe LED current

breadboard/polarity

INPUT_PULLUP and debounce

PWM duty cycle and protected loads

## Lab 3: Safe LED Circuit and Ohm's Law

| Alignment | LO2 |
| --- | --- |
| Time | 10 minutes |
| Objective | Calculate and build a current-limited LED circuit within an Arduino digital pin's electrical boundary. |
| Deliverable | A documented 5 V digital-output circuit using a 330 ohm resistor and measured LED voltage. |
| Folder | labs/lab-03-safe-led-circuit/ |

### Components

Arduino Uno or Maker UNO

Breadboard

LED

330 ohm resistor

Jumper wires

Multimeter

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| D7 | 330 ohm resistor | Series current limit |
| Resistor | LED anode | Long lead |
| LED cathode | GND | Short lead / flat edge |

### Detailed procedure

Calculate R=(5 V-2 V)/0.01 A and explain why 330 ohm is a safe standard value.

Build the series path D7 -> resistor -> LED -> GND with power disconnected.

Upload the sketch and verify the LED toggles once per second.

Measure LED forward voltage while ON and calculate the observed current from the resistor voltage.

### Solution sketch

const uint8_t LED_PIN = 7; void setup() { pinMode(LED_PIN, OUTPUT); } void loop() { digitalWrite(LED_PIN, HIGH); delay(500); digitalWrite(LED_PIN, LOW); delay(500); }

### Acceptance checks

LED polarity is correct

A current-limiting resistor is in series

LED toggles at 1 Hz

Calculated current remains close to or below 10 mA

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| LED never lights | Reverse the LED and confirm D7 is connected through the resistor to the anode. |
| LED very dim | Measure the supply and forward voltage; confirm the resistor is 330 ohm, not 330 kilohm. |
| Board resets | Disconnect immediately and check for a short between 5 V and GND. |

## Lab 4: Push Button with INPUT_PULLUP

| Alignment | LO2 |
| --- | --- |
| Time | 10 minutes |
| Objective | Eliminate a floating input and debounce a mechanical switch using a stable time window. |
| Deliverable | An active-LOW button that toggles an LED once per physical press without double triggers. |
| Folder | labs/lab-04-button-input-pullup/ |

### Components

Arduino Uno or Maker UNO

Push button

LED circuit from Lab 3

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| D2 | Push button terminal A | INPUT_PULLUP |
| Push button terminal B | GND | Pressed = LOW |
| D7 | LED + 330 ohm | Output evidence |

### Detailed procedure

Place the button across the breadboard centre gap and identify the internally common legs.

Wire D2 to one side and GND to the opposite side; do not add an external pull-down.

Upload the sketch and press/release the button repeatedly at different speeds.

Observe the raw state in Serial Monitor and confirm one LED toggle per accepted press.

### Solution sketch

const uint8_t BUTTON_PIN = 2; const uint8_t LED_PIN = 7; const unsigned long DEBOUNCE_MS = 30; bool ledOn = false; bool stableState = HIGH; bool lastRaw = HIGH; unsigned long changedAt = 0; void setup() { pinMode(BUTTON_PIN, INPUT_PULLUP); pinMode(LED_PIN, OUTPUT); } void loop() { bool raw = digitalRead(BUTTON_PIN); if (raw != lastRaw) { changedAt = millis(); lastRaw = raw; } if (millis() - changedAt >= DEBOUNCE_MS && raw != stableState) { stableState = raw; if (stableState == LOW) { ledOn = !ledOn; digitalWrite(LED_PIN, ledOn ? HIGH : LOW); } } }

### Acceptance checks

Released input reads HIGH

Pressed input reads LOW

LED toggles once per press

Bounce within 30 ms does not create extra toggles

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Input always LOW | Check that D2 and GND are not on the same internally connected button leg. |
| Random triggers | Confirm INPUT_PULLUP is used and all grounds are common. |
| Double toggles | Increase DEBOUNCE_MS slightly and ensure only the falling edge toggles state. |

## Lab 5: PWM Fade with Non-blocking Timing

| Alignment | LO2 |
| --- | --- |
| Time | 15 minutes |
| Objective | Relate 8-bit PWM values to duty cycle and vary LED brightness without blocking other logic. |
| Deliverable | A triangular 0-255 PWM ramp on D9 with calculated duty-cycle checkpoints. |
| Folder | labs/lab-05-pwm-fade/ |

### Components

Arduino Uno or Maker UNO

LED

330 ohm resistor

Breadboard

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| D9 (~) | 330 ohm resistor | PWM output |
| Resistor | LED anode | Series |
| LED cathode | GND | Return |

### Detailed procedure

Move the LED circuit to PWM-capable pin D9 and confirm the tilde marking/pin map.

Upload the non-blocking fade sketch and monitor the brightness value at key checkpoints.

Calculate duty cycle for values 64, 128, and 192 using value/255 x 100%.

Change STEP to 15 and compare the smoothness and cycle duration.

### Solution sketch

const uint8_t LED_PIN = 9; const unsigned long UPDATE_MS = 20; const int STEP = 5; int brightness = 0; int direction = STEP; unsigned long lastUpdate = 0; void setup() { pinMode(LED_PIN, OUTPUT); } void loop() { if (millis() - lastUpdate < UPDATE_MS) return; lastUpdate = millis(); brightness += direction; if (brightness >= 255) { brightness = 255; direction = -STEP; } if (brightness <= 0) { brightness = 0; direction = STEP; } analogWrite(LED_PIN, brightness); }

### Acceptance checks

LED fades up and down continuously

No delay() is used in loop()

Value never exceeds 0-255

Calculated 128 duty cycle is approximately 50%

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| No fade | Confirm the LED is on a PWM pin such as D9, not a non-PWM pin. |
| Brightness jumps | Reduce STEP or shorten UPDATE_MS. |
| Value wraps | Reverse direction before the integer crosses the 0 or 255 bounds. |

# Topic 3: Analogue Sensors and Transducers

Analogue control starts with a continuous quantity and ends with a bounded digital decision. The 10-bit ADC quantises voltage into 0-1023, calibration maps real endpoints, filtering reduces noise, hysteresis stabilises threshold control, and servo power must remain separate from the signal pin.

## Slide alignment

Slides 47-64 | Skill focus: Apply analogue sensors and transducers to solve real-world control problems | Key concepts: Applications of analogue sensors and transducers

## Mechanism checklist

ADC conversion and resolution

calibration/filtering

LDR divider and hysteresis

servo mapping and power

## Lab 6: LDR Dark-Activated Lamp with Hysteresis

| Alignment | LO3 |
| --- | --- |
| Time | 15 minutes |
| Objective | Convert a light-dependent voltage divider into a stable two-threshold control decision. |
| Deliverable | A calibrated dark-activated lamp that avoids chatter around the switching threshold. |
| Folder | labs/lab-06-ldr-dark-lamp/ |

### Components

Arduino Uno or Maker UNO

LDR

10 kilohm resistor

LED + 330 ohm

Breadboard

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| 5 V | LDR | Divider top |
| LDR/resistor junction | A0 | ADC input |
| 10 kilohm resistor | GND | Divider bottom |
| D7 | LED circuit | Lamp output |

### Detailed procedure

Build the divider and record A0 values under bright, room, and covered conditions.

Choose ON_THRESHOLD below the dark value and OFF_THRESHOLD above it to create a dead band.

Upload the solution and slowly shade the LDR while observing state and ADC evidence.

Explain why one threshold would chatter when readings contain noise.

### Solution sketch

const uint8_t LDR_PIN = A0; const uint8_t LAMP_PIN = 7; const int ON_THRESHOLD = 350; const int OFF_THRESHOLD = 450; bool lampOn = false; void setup() { pinMode(LAMP_PIN, OUTPUT); Serial.begin(9600); } void loop() { int light = analogRead(LDR_PIN); if (!lampOn && light < ON_THRESHOLD) lampOn = true; if (lampOn && light > OFF_THRESHOLD) lampOn = false; digitalWrite(LAMP_PIN, lampOn ? HIGH : LOW); Serial.println(light); delay(100); }

### Acceptance checks

Serial output shows plausible 0-1023 readings

Lamp turns ON in dark conditions

Lamp turns OFF in bright conditions

Output remains stable inside the hysteresis band

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Reading fixed at 0 | Check A0 is not shorted to GND and the LDR is connected to 5 V. |
| Reading fixed at 1023 | Check the 10 kilohm resistor reaches GND. |
| Logic reversed | Swap divider orientation or reverse the comparison to match measured calibration. |

## Lab 7: Potentiometer-Controlled Servo

| Alignment | LO3 |
| --- | --- |
| Time | 15 minutes |
| Objective | Map a 10-bit ADC measurement to a bounded servo command while respecting the servo power boundary. |
| Deliverable | A 0-180 degree servo position controller with clamped input and separate-power guidance. |
| Folder | labs/lab-07-pot-servo/ |

### Components

Arduino Uno or Maker UNO

10 kilohm potentiometer

SG90 servo

External regulated 5 V supply recommended

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| Pot outer pins | 5 V and GND | Divider |
| Pot wiper | A0 | ADC input |
| Servo signal | D9 | Control pulse |
| Servo V+ | Regulated 5 V | Do not overload I/O pin |
| All grounds | Common GND | Signal reference |

### Detailed procedure

Wire the potentiometer divider and confirm A0 spans most of 0-1023.

Connect the servo signal to D9 and establish a common ground with its power source.

Upload the solution, rotate slowly, and verify commanded angle follows the knob.

Test the endpoints and confirm constrain() prevents commands outside 0-180 degrees.

### Solution sketch

#include <Servo.h> Servo actuator; const uint8_t POT_PIN = A0; const uint8_t SERVO_PIN = 9; void setup() { actuator.attach(SERVO_PIN); Serial.begin(9600); } void loop() { int raw = analogRead(POT_PIN); int angle = constrain(map(raw, 0, 1023, 0, 180), 0, 180); actuator.write(angle); Serial.println(angle); delay(15); }

### Acceptance checks

ADC reading changes smoothly

Servo follows the potentiometer

Command stays within 0-180 degrees

Board does not reset when servo moves

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Servo jitters | Improve the 5 V supply, common the grounds, and add a capacitor near the servo. |
| Servo moves backward | Reverse the map output range if required by the application. |
| Board resets | Power the servo from a suitable external 5 V source rather than an I/O pin. |

# Topic 4: Digital Sensors and Transducers

Digital sensors encode information as levels, pulse widths or framed data. Reliable control requires correct thresholds, edges, timeouts and explicit fault states. Ultrasonic distance uses round-trip time of flight; PIR motion produces an active-level contract combined with a non-blocking hold timer.

## Slide alignment

Slides 65-83 | Skill focus: Apply digital transducers to solve real-world control problems | Key concepts: Applications of digital transducers

## Mechanism checklist

digital output contracts

ultrasonic trigger/echo and timeout

distance formula

PIR state and hold timing

## Lab 8: Ultrasonic Distance with Timeout

| Alignment | LO4 |
| --- | --- |
| Time | 15 minutes |
| Objective | Measure echo pulse width, convert time-of-flight to distance, and handle a missing echo safely. |
| Deliverable | A distance monitor that reports centimetres or NO_ECHO within a bounded 30 ms wait. |
| Folder | labs/lab-08-ultrasonic-distance/ |

### Components

Arduino Uno or Maker UNO

HC-SR04/HY-SRF05 ultrasonic sensor

Breadboard

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| Sensor VCC | 5 V | Supply |
| Sensor GND | GND | Return |
| TRIG | D9 | 10 microsecond output pulse |
| ECHO | D10 | Pulse input |

### Detailed procedure

Wire the sensor and confirm the trigger and echo pins are not swapped.

Upload the sketch and position a flat target 20-100 cm in front of the sensor.

Compare Serial Monitor distance with a ruler at three positions.

Remove the target and confirm the 30 ms timeout returns NO_ECHO instead of blocking forever.

### Solution sketch

const uint8_t TRIG_PIN = 9; const uint8_t ECHO_PIN = 10; const unsigned long ECHO_TIMEOUT_US = 30000; void setup() { pinMode(TRIG_PIN, OUTPUT); pinMode(ECHO_PIN, INPUT); Serial.begin(9600); } void loop() { digitalWrite(TRIG_PIN, LOW); delayMicroseconds(2); digitalWrite(TRIG_PIN, HIGH); delayMicroseconds(10); digitalWrite(TRIG_PIN, LOW); unsigned long duration = pulseIn(ECHO_PIN, HIGH, ECHO_TIMEOUT_US); if (duration == 0) Serial.println("NO_ECHO"); else { float distanceCm = duration * 0.0343f / 2.0f; Serial.println(distanceCm, 1); } delay(100); }

### Acceptance checks

Distance is within a reasonable tolerance of the ruler

Readings update continuously

Missing echo returns NO_ECHO

No divide-by-zero or indefinite wait occurs

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Always NO_ECHO | Swap TRIG/ECHO if reversed and verify the sensor has 5 V and common GND. |
| Distance doubles | The conversion must divide the round-trip distance by two. |
| Unstable values | Use a flat target, avoid soft fabric, and keep within the sensor's working zone. |

## Lab 9: PIR Motion-Activated Light

| Alignment | LO4 |
| --- | --- |
| Time | 10 minutes |
| Objective | Use a digital motion sensor contract and a non-blocking hold timer to control a light. |
| Deliverable | A motion light that extends its ON period when new motion arrives and expires safely after five seconds. |
| Folder | labs/lab-09-pir-motion-light/ |

### Components

Arduino Uno or Maker UNO

HC-SR501 PIR sensor

LED + 330 ohm

Breadboard

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| PIR VCC | 5 V | Supply |
| PIR GND | GND | Return |
| PIR OUT | D2 | Digital input |
| D7 | LED circuit | Light output |

### Detailed procedure

Wire the PIR and allow its warm-up/calibration interval after power-up.

Upload the sketch and move across the sensor field of view.

Trigger motion again before five seconds expires and confirm the hold timer extends.

Remain still and verify the output turns OFF after the bounded hold period.

### Solution sketch

const uint8_t PIR_PIN = 2; const uint8_t LIGHT_PIN = 7; const unsigned long HOLD_MS = 5000; unsigned long lastMotionMs = 0; void setup() { pinMode(PIR_PIN, INPUT); pinMode(LIGHT_PIN, OUTPUT); } void loop() { if (digitalRead(PIR_PIN) == HIGH) lastMotionMs = millis(); bool active = lastMotionMs != 0 && millis() - lastMotionMs < HOLD_MS; digitalWrite(LIGHT_PIN, active ? HIGH : LOW); }

### Acceptance checks

Motion drives the output HIGH

Output remains ON for five seconds after the latest motion

New motion extends the timer

loop() remains responsive without delay(5000)

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Always HIGH | Wait for warm-up and check the sensor retrigger jumper and sensitivity settings. |
| No detection | Confirm OUT reaches D2 and walk laterally across the field of view. |
| Light turns off too soon | Update lastMotionMs every time the PIR is HIGH. |

# Topic 5: Actuators

Actuators consume energy and therefore require a power stage. MOSFETs or driver arrays switch load current, flyback paths clamp inductive energy, step sequences create position, and feedback controllers validate sensor readings before applying hysteresis and commanding the actuator.

## Slide alignment

Slides 84-102 | Skill focus: Develop actuator networks with stepper and continuous-drive actuators | Key concepts: Actuator networks

## Mechanism checklist

driver energy boundary

DC motor/flyback protection

stepper phases and position

DHT validity and feedback hysteresis

## Lab 10: DHT11 Temperature and Humidity Monitor

| Alignment | LO5 |
| --- | --- |
| Time | 10 minutes |
| Objective | Read a library-managed digital sensor transaction and reject invalid measurements before control use. |
| Deliverable | A one-hertz temperature/humidity monitor that identifies NaN failures instead of treating them as real values. |
| Folder | labs/lab-10-dht-monitor/ |

### Components

Arduino Uno or Maker UNO

DHT11 module

10 kilohm pull-up if using a bare sensor

Jumper wires

Adafruit DHT sensor library

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| DHT VCC | 5 V | Supply |
| DHT GND | GND | Return |
| DHT DATA | D2 | Digital data |
| DATA to VCC | 10 kilohm | Pull-up for bare sensor |

### Detailed procedure

Install the Adafruit DHT sensor library and its required dependency from Library Manager.

Wire the DHT11 and upload the monitor sketch.

Warm the sensor gently and observe the change without touching exposed conductors.

Disconnect DATA briefly and confirm the sketch reports READ_ERROR rather than a numeric control value.

### Solution sketch

#include <DHT.h> const uint8_t DHT_PIN = 2; #define DHT_TYPE DHT11 DHT dht(DHT_PIN, DHT_TYPE); void setup() { Serial.begin(9600); dht.begin(); } void loop() { float t = dht.readTemperature(); float h = dht.readHumidity(); if (isnan(t) ¦¦ isnan(h)) Serial.println("READ_ERROR"); else { Serial.print("T_C="); Serial.print(t, 1); Serial.print(",RH="); Serial.println(h, 1); } delay(2000); }

### Acceptance checks

Temperature and humidity update approximately once per second

Values are plausible for the room

Disconnected/failed read reports READ_ERROR

dht.begin() is called in setup()

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| READ_ERROR | Check sensor type, D2 wiring, pull-up, common ground, and the two-second minimum sample interval if needed. |
| Values never change | Avoid reading faster than the sensor supports and confirm the correct DHT type. |
| Compile error | Install both the DHT sensor library and Adafruit Unified Sensor dependency. |

## Lab 11: Stepper Motor Position Control

| Alignment | LO5 |
| --- | --- |
| Time | 10 minutes |
| Objective | Drive a four-phase stepper through a transistor array and relate commanded steps to angular position. |
| Deliverable | A 28BYJ-48/ULN2003 actuator that moves one revolution forward and backward at a bounded speed. |
| Folder | labs/lab-11-stepper-position/ |

### Components

Arduino Uno or Maker UNO

28BYJ-48 stepper

ULN2003 driver board

External 5 V supply

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| D8-D11 | ULN2003 IN1-IN4 | Phase controls |
| ULN2003 5 V | External 5 V | Motor power |
| ULN2003 GND | Arduino GND | Common reference |
| Stepper | ULN2003 socket | Keyed connector |

### Detailed procedure

Connect the driver inputs in the library sequence and establish common ground.

Upload the sketch with STEPS_PER_REV set for the supplied motor/gearbox.

Mark the shaft, observe one forward and one reverse revolution, and record position error.

Increase speed modestly and identify the point where the motor begins to miss steps.

### Solution sketch

#include <Stepper.h> const int STEPS_PER_REV = 2048; Stepper motor(STEPS_PER_REV, 8, 10, 9, 11); void setup() { motor.setSpeed(10); } void loop() { motor.step(STEPS_PER_REV); delay(500); motor.step(-STEPS_PER_REV); delay(500); }

### Acceptance checks

Motor completes a forward and reverse move

Direction reverses correctly

Driver—not Arduino I/O—supplies coil current

Grounds are common and board remains stable

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Motor vibrates | Check IN1-IN4 order and use the phase order required by the library/driver. |
| Weak torque | Use a suitable external 5 V supply and lower the speed. |
| Wrong angle | Calibrate STEPS_PER_REV for the actual gearbox and stepping mode. |

## Lab 12: DHT Fan Controller with Hysteresis

| Alignment | LO5 |
| --- | --- |
| Time | 10 minutes |
| Objective | Integrate measurement, decision logic, and a protected continuous-drive actuator into a feedback controller. |
| Deliverable | A temperature-controlled DC fan with MOSFET switching, flyback protection, PWM drive, and hysteresis. |
| Folder | labs/lab-12-dht-fan-controller/ |

### Components

Arduino Uno or Maker UNO

DHT11

Logic-level N-channel MOSFET

DC fan/motor

Flyback diode

External motor supply

Jumper wires

### Wiring contract

| From | To | Purpose |
| --- | --- | --- |
| DHT DATA | D2 | Measurement |
| D5 PWM | MOSFET gate through 220 ohm | Control |
| 10 kilohm | Gate to GND | Safe default OFF |
| Motor/fan | External supply + MOSFET drain | Power path |
| Flyback diode | Across motor | Cathode to supply + |
| All grounds | Common GND | Reference |

### Detailed procedure

Build the low-side MOSFET driver with power disconnected and verify diode polarity.

Upload the controller and confirm the fan is OFF below FAN_ON_C.

Warm the DHT11 above the ON threshold and observe full PWM drive.

Let the sensor cool below FAN_OFF_C and confirm the fan does not chatter between thresholds.

### Solution sketch

#include <DHT.h> const uint8_t DHT_PIN = 2; const uint8_t FAN_PWM_PIN = 5; const float FAN_ON_C = 30.0; const float FAN_OFF_C = 28.0; DHT dht(DHT_PIN, DHT11); bool fanOn = false; void setup() { pinMode(FAN_PWM_PIN, OUTPUT); Serial.begin(9600); dht.begin(); } void loop() { float t = dht.readTemperature(); if (isnan(t)) { Serial.println("READ_ERROR"); delay(2000); return; } if (!fanOn && t >= FAN_ON_C) fanOn = true; if (fanOn && t <= FAN_OFF_C) fanOn = false; analogWrite(FAN_PWM_PIN, fanOn ? 255 : 0); Serial.print("T_C="); Serial.print(t, 1); Serial.print(",FAN="); Serial.println(fanOn ? "ON" : "OFF"); delay(2000); }

### Acceptance checks

Fan turns ON above 30 C

Fan remains ON until temperature falls below 28 C

Invalid DHT read leaves the previous safe state and reports an error

Motor current never flows through an Arduino I/O pin

### Troubleshooting

| Symptom | Diagnostic / correction |
| --- | --- |
| Fan never starts | Check external supply, common ground, MOSFET pinout, and gate drive. |
| Arduino resets | Separate motor power, add decoupling, and verify flyback diode orientation. |
| Fan chatters | Use distinct ON/OFF thresholds and a realistic sample interval. |

# Technical References

Arduino language reference: https://docs.arduino.cc/language-reference/

Arduino PWM Help Center: https://support.arduino.cc/hc/en-us/articles/9350537961500-Use-PWM-output-with-Arduino

Course registration: https://www.tertiarycourses.com.sg/arduino-masterclass.html

Legacy source deck: Reference v12 PPTX supplied with this project